The WidowX robot arm that has been used for the validation of the computing platform has six degrees of freedom corresponding to each one of its actuator joints as shown in figure below:

The physical model is built based on the Denavit-Hartenberg parametrization, which provides an standard way of attaching reference frames from the physical links of a robot manipulator. The joints are selected depending on the degrees of freedom of the arm and constructional constraints. For the elaboration of the model, five joints are identified:

- Joint 1: Servo 1

- Joint 2: Servo 2

- Joint 3: Elbow (fixed 90º due to arm structure)

- Joint 4: Servo 3

- Joint 5: Servo 4

Since servos 5 and 6 do not affect to the final position of the grip (only to its orientation and to the gripping movement) neither they intervene in the physical model equations, nor they are considered when sending the commands to the arm.

Knowing the z-axis for each frame matches the joint axis, these parameters are defined as:

- J: number of joint, i.e. index;

- r: value of the radius about previous z-axis;

- d: offset of the origin of the reference frame along previous z-axis;

- α: angle between the z-axis of previous and current joint.

A diagram of simple kinematic chain representing these parameters is presented in Error: Reference source not found (note that designation ‘a’ is used instead of ‘d’ in the figure):

Table below shows the Denavit-Hartenberg convention parameters for the model of the robot:

| Ji | ri | di | αi | Θi |
| --- | --- | --- | --- | --- |
| 1 | 0 | 0 | π/2 | Θ1 |
| 2 | 15 | 0 | -π | Θ2 |
| 3 | 5 | 0 | 0 | Θ3 |
| 4 | 15 | 0 | 0 | Θ4 |
| 5 | 15 | 0 | 0 | Θ5 |

Some of these parameters may be chosen differently, but after considering the direction of the rotation of each servo the resulting transformation equations should be equivalent. The correctness of the physical model was ensured by obtaining a 3-D representation of the arm based on calculated parameters. A view of this three-dimensional representation generated by the Peter Corke toolbox is depicted in figure below: