In robotics, many times it is interesting to know what is the set of locations that can be reached. This is what is called the “workspace” of the robot and depends on constructional parameters of the robot itself, but also on the end-effector mounted. The workspace for a six-axis robotic arm is a six-dimensional space that consists of all possible combination of values for the six degrees of freedom of the arm, i.e. the complete set of positions and orientations of the robot.

Also, singularities are a very important phenomenon that limit the usable parts of the workspace. A singularity is a position where IK fail, and the robot end-effector becomes blocked in certain directions. Moreover, passing close to a singularity results in high joint velocities which may be unexpected and may be dangerous for the robot itself and its surroundings. Main singularities have been identified (wrist, elbow and shoulder singularity) for wrist-partitioned, vertically articulated robotic arms, but these occurrences can only be avoided through the appropriate design of the whole robot cell.

Typically, the workspace is a combination of several singularity-free workspace subsets. For this use case this analysis has not been performed since it is outside the scope of the project. A “safe zone” has been specified instead in cylindrical coordinates, where the robot can be moved without the risk of encountering a singularity if specific guidelines are followed. The 3D representation of this safe zone is depicted in figure below:

1+25ρ<z<29−25ρ10<ρ<35(ρ2=x2+y2)

An additional consideration when moving from point A(x0,y0,z0) to B (x1,y1,z1) is to guarantee that at least sgn(x1)=sgn(x0) or sgn(y1)=sgn(y0), i.e.  not direct displacement is produced from one quadrant of the XY plane to the opposite one.

Following this recommendations the singularities should be avoided and safe trajectories are expected to be executed each time.