The Nelder-Mead optimization method is often used in non-linear searches of the minimum or maximum of a cost function since it is a direct search method that does not need the calculation of the function derivatives.

The main drawback of this method is that convergence is not mathematically guaranteed, and the optimization process could lead to non-stationary points. However, after extensive engineering testing it has not given convergence problems once for this application, being capable of finding a solution every time in a low number of iterations and proven itself to be a valuable tool for the generation of trajectories.

The optimization process looks for a local optimum of a multidimensional problem, finding the approximate solution when the cost function varies smoothly. The accuracy of the searching process can be customized and that will impact on the number of iterations, but given a required accuracy the length of the iterative process cannot be known in advance.

This mechanism starts with a 6-dimensional simplex: in this scenario, each dimension represents a degree of freedom of the arm. The centroid of the starting simplex is determined by the joint angles of the starting position (which are known).

Then, a series of transformations are applied to this simplex as seen in figure belowError: Reference source not found:

The cost function is evaluated in all vertices and then, a transformation in the simplex is produced. Depending on the values of the simplex, it will experience expansions, contractions or reflections in order to narrow the search more and more after each iteration.

A contingency mechanism is implemented, in such a way that if the solution is not found in a number of iterations, the initial simplex is recalculated with random values generated around the starting angles, and the process starts over.

In this scenario, the cost function is calculated as follows: first, the forward kinematics equations are applied to the vertex in order to obtain its corresponding Cartesian coordinates. Then, the least squares fitting between this point and the target point is calculated. In this step, the constraint of canceling the z coordinate of two last joints is added in order to keep the last section of the arm straight and avoid weird positions during the movement. When the cost function value is smaller than the required accuracy, the search process concludes and the angles of the joints for the final position are provided.

This process will need to be repeated a number of times until reach the final position since, in order to avoid jerky movements and increase its smoothness, the algorithm works with a list of intermediate points that could be generated through linear interpolation or any other method.