# Widow X robotic arm GUI command protocol

All commands use encoding ‘ascii’.

Initialization
Command: “\n\r”
Description: It executes function widowx_start() to carry the arm to know position (35, 0, 15).

Set final point
Command: “FP\n” followed by “XFxYFxZF\n”, where (XF, YF, ZF) are the final coordinates for the tip position of the arm.
Description: Moves the arm to (XF, YF, ZF).

Set final point + intermediate point
Command: “IP\n” followed by “XFxYFxZF\n” and followed by “XIxYIxZI\n”, where (XF, YF, ZF) are the final coordinates for the tip position of the arm and (XI, YI, ZI) is the required intermediate point.
Description: Moves the arm to (XF, YF, ZF) passing through (XI, YI, ZI).

Preset trajectories
Command: “SETX\n”, where X is the number of point set to be executed.
Description: Executes preset trajectory in loop stored in internal file trjX.txt.

Sense solar storm
Command: “SOL1\n”
Description: Commands the computing platform to activate redundancy.

Start solar storm
Command: “SOL2\n”
Description: Commands the computing platform to activate failure emulation.

End solar storm
Command: “SOL3\n”
Description: Commands the computing platform to deactivate redundancy and failure emulation.

Start sand storm
Command: “SAND1\n”
Description: Commands the computing platform to enter low energy mode (reduce HW acceleration”).

End sand storm
Command: “SAND2\n”
Description: Commands the computing platform to exit low energy mode (increase HW acceleration”).

Set accuracy
Command: “AX\n”, wehe X is the required accuracy of the activity.
Description: Sets the value of required accuracy in internal file acc.txt to X.